2. Fundamental assumptionsThe basic assumptions of the constitutive and computational modeling, as well as related limitations, are summarized as follows:

- Only the configuration of deep tunnels shall be considered in the subsequent analysis, thus neglecting deformations caused by surface loads and settlements arising from the excavation process;

- Although material heterogeneity and behavior anisotropy are inherent features of soils and rocks, the rock mass is modeled throughout the paper as a homogeneous and isotropic continuous medium. At the scale adopted for tunnel modeling (macroscopic scale), this assumption means in particular that the possible micro-heterogeneities, such isotropic distributions of joints or cracks present at the finer scale, are accounted for in the homogenized behavior by means of a preliminary homogenization process (e.g., [20, 14, 7, 19, 1]). Clearly enough, the framework of continuum modeling adopted in the paper would reveal questionable when the rock mass is cut by a few macroscale fracture joints;

- The rock mass is phenomenologically modeled using an elastoplastic-viscoplastic rheological law to capture instantaneous and long-term responses. This approach disregards the aspect connected temperature gradients, water flow, and poromechanics coupling;

- Despite the complexity of the stress distribution prevailing in the rock mass before the process of tunnel excavation, which is mainly affected by the geological history, the present study assumes a geostatic initial stress reflected by an isotropic state of stress.

- Twin tunnels are often designed considering a time gap between excavation fronts. However, the finite element simulations assume synchronous excavation steps to ensure symmetry conditions.

- The simulation excavation processes are curried out assuming a constant tunnel advancement rate (i.e., constant excavation speed), together with a constant thickness of concrete lining.

- Effects of temperature and humidity that may affect the viscoelastic behavior of lining concrete are disregarded.

- Perfect bonding is assumed at the interface between concrete lining and the rock mass.

- The framework of infinitesimal strain analysis, together with quasi-static evolutions, is adopted in the paper. In particular, dynamic excitations and related inertial forces, such as those induced, for instance, by earthquakes or explosions, shall not be considered in the numerical analysis.

3. Constitutive Model of the Rock MaterialTime-dependent phenomena associated with the delayed behavior of the constitutive material are key aspects of deformation in tunnel structures excavated in deep clayey rocks (see for instance [29, 21] or [16], to cite a few). In most computational analyses developed for tunnel engineering design, this issue is generally addressed by means of viscoplastic constitutive behavior. While such constitutive models could relevantly model the transient and long-term deformation, they seem however inadequate to capture the influence of short-term events (tunnelling and support placement phases) on the final stability of the structure. In particular, an analysis of tunnel deformation based on a viscoplastic model would suggest that the ultimate support pressure at tunnel structure equilibrium mainly depends on the closure rate at the moment when the contact between lining and rock mass is achieved (e. g., [21]), thus disregarding the irreversible effects rising in the initial construction phases. Indeed, during the primary stages of tunnel excavation, the surrounding rock mass is subjected to severe loading conditions and hight strain rates, which may lead to yielding associated with high instantaneous irreversible strains near the tunnel wall, and can therefore affect the long-term equilibrium of the structure. It is thus of fundamental concern to formulate a constitutive model that incorporates both instantaneous and delayed irreversible components of the rock material. For this purpose, the present analysis considers a constitutive model that includes both instantaneous plasticity to describe shorth-term material yielding and viscoplasticity to represent delayed behavior. The formulation of the coupled plasticity-viscoplasticity rheological model is based on that originally proposed in [21] and [29]. Previous studies have implemented this plastic-viscoplastic model for computational analysis of deformation in single tunnels (e.g., [6, 23, 16, 25]. For the sake of brevity, only the main features of this constitutive model shall be summarized below. Detailed description of the model, including application and validation in the context of single tunnel structures may be found in [27]. Finite element implementation of this model in the USERMAT procedure of ANSYS software is also described in [25].

The elastoplastic-viscoplastic model is formulated based on a serial association of the elastoplastic and viscoplastic constitutive models. The local strain rate 𝜺̇ is split into three contributions 𝜺̇ = 𝜺̇ 𝑒 + 𝜺̇ 𝑝 + 𝜺̇ 𝑣𝑝, so that the constitutive relationships relating the Cauchy stress rate 𝝈̇ and strain rate components can be written as:

𝝈̇ = 𝑫 ∶ 𝜺̇ 𝑒 = 𝑫 ∶ (𝜺̇ − 𝜺̇ 𝑝 − 𝜺̇ 𝑣𝑝).	(1)

In the above relationship, 𝜺̇ 𝑒, 𝜺̇ 𝑝 and 𝜺̇ 𝑣𝑝, represent respectively the elastic, plastic and viscoplastic strain rate, and 𝑫

denote the fourth-order isotropic elastic linear constitutive tensor. The one-dimensional representation in Fig. 1 shows this association. In this model is used a Drucker-Prager plastic flow surface given by

Figure 1: Rheological representation of the elastoplastic-viscoplastic model.

√	(2)

𝑓 (𝝈, 𝑞) = 𝑓 (𝐼1, 𝐽2, 𝑞) = 𝛽1𝐼1 + 𝛽2 𝐽2 − 𝑞(𝛼), which 𝐼1 is the first invariant of the stress tensor, 𝐽2 the second invariant of the deviator tensor and 𝛽1, 𝛽2 and 𝑞(𝛼) are strength parameters related to the friction angle 𝜙 and cohesion 𝑐(𝛼), respectively. In the present model Drucker-Prager surface been inner of the Mohr-Coulomb surface [5], that is,

𝛽1 =  ,	𝛽,	(3)

where 𝑘 = (1 + sin 𝜙)∕(1 − sin 𝜙). The internal variable 𝛼 is the equivalent plastic strain 𝜀̄𝑝 used to simulate strain hardening/softening phenomena. However, for this study, we adopt perfect plasticity, meaning that c is a constant. For the viscoplasticity surface 𝑣𝑝 the same surface is empolyed, but with 𝑣𝑝 in and , and 𝑣𝑝 √ 𝑣𝑝 where

𝑓	𝜙	𝛽1	𝛽2	𝑞	= 2	𝑘	− 𝑐𝑣𝑝

𝑘𝑣𝑝 = (1 + sin 𝜙𝑣𝑝)∕(1 − sin 𝜙𝑣𝑝) and 𝑐𝑣𝑝 is a constant, i.e., perfect viscoplasticity. The plastic flow rule is given by:

# {	𝜕𝑔

̇𝑝 =	𝜆̇ 𝜕𝝈	forfor 𝑓 ≤> 0 ,	(4)

## 𝜺

### 𝟎,	𝑓	0

where 𝜆̇ is the plasticity multiplier and 𝑔 is a potential flow function analogous to 𝑓 used to simulate the volume dilatation during the evolution of plastic deformations. However, for this analysis, was used associated plasticity, i.e., 𝑔 = 𝑓 . The plastic multiplier is obtained through the consistency condition 𝑓̇ = 0. Numerical details of this implementation can be found in [27]. For viscoplastic flow rule we have,

𝑣𝑝	𝜕𝑓 𝑣𝑝

𝜺̇	= 𝜆̇ 𝑣𝑝		(5)

## 𝜕𝝈

In contrast to the plastic multiplier, the viscoplastic multiplier 𝜆𝑣𝑝 is independent of a consistency like condition. As a result, its expression is explicit. Based on the framework of generalized Perzyna’s overstress theory [22], its expression may be derived as follows:

𝑣𝑝	Φ(𝝈, 𝑞𝑣𝑝)	and Φ = ⟨ 𝑓 𝑣𝑝(𝑓𝝈0, 𝑞𝑣𝑝)⟩𝑛 ,	(6)

### 𝜆̇	=

𝜂 where Φ is the overstress function, 𝜂 is the dynamic viscosity constant, 𝑛 is the dimensionless parameter that gives the form of the power law, 𝑓0 a parameter conveniently adopted and ⟨∗⟩ is the McCauley function which is 0 when ∗< 0 , i.e. viscoplastic flow will only occur when the overstress function is positive.

In this coupled model, when 𝜙 = 𝜙𝑣𝑝, cohesion entirely controls the evolution of local mechanical fields. Specifically, when 𝑐 → ∞ and 𝑐𝑣𝑝 → ∞, the system achieves a purely elastic solution. The solution becomes purely elastoviscoplastic with 𝑐 → ∞, while a pure elastoplastic solution emerges with 𝑐𝑣𝑝 → ∞. In the coupled analysis, condition 𝑐𝑣𝑝 < 𝑐 is adopted, allowing the viscoplastic domain to occur without plasticity. However, in the presence of plasticity, viscous effects become inevitable. Fig. 2 illustrates these domains in principal stress space.

Elastic domain

Viscoplastic surface

Plastic surface

Elastic-viscoplastic domain Elastic-plastic-viscoplastic domain

Figure 2: Elastoplastic-viscoplastic domains.

## 4. Constitutive Model of the Lining

Shrinkage and creep phenomena represent fundamental components of concrete deformation processes that are expected to naturally affect the instantaneous as well as the transient and long-term behavior of structures involving such material. However, most of the tunnel design analyses consider the concrete involved in lining systems as a linear elastic material. From a phenomenological point of view, creep of concrete refers to the time-dependent deformation induced by sustained loading, whereas shrinkage deformation refers to the volume decrease caused by drying. As far as deformation in tunnel structures is concerned, creep and shrinkage have an important effect on the performance of the concrete lining and consequently on its contribution to controlling the long-term convergence of the tunnel. To account for such constitutive features, the concrete creep deformation is addressed by means of an aging viscoelastic rheological model relying on Bažant and Prasannan Solidification Theory [3, 4]. The viscoelastic model is described by a Generalized Kelvin chainl as depicted in Fig. 3. The mechanical parameters that define such a rheological model are the springs stiffness and dash-pots viscosity.    These model parameters are calibrated based on the CEB-FIP MC90 standard specifications formulation reported in [8]. One may refer to [26, 28] for detailed description of the calibration procedure. As regards the concrete deformation associated with shrinkage, the isotropic formulation proposed in CEB-FIP MC90 standard [8] is adopted in the present modeling and subsequent computational analyses. Full details regarding model definition and related finite element implementation may be found in [24] and [26].

Accordingly, the constitutive equations for concrete lining relating the stress and strain rate can be expressed in the

framework of infinitesimal strain analysis as:

𝝈̇ = 𝑫 ∶ 𝜺̇ 𝑒 = 𝑫 ∶ 𝜺̇ − 𝑫 ∶ 𝜺̇ 𝑠ℎ − 𝑫∗ ∶ 𝜺̇ 𝑐𝑟	(7)

Figure 3: Generalized Kelvin model for concrete viscoelasticity

In the above relationship, 𝜺̇ 𝑠ℎ and 𝜺̇ 𝑐𝑟 are respectively the shrinkage and creep strain rates. The fourth-order tensors 𝑫 and 𝑫∗ refer to the isotropic elastic linear constitutive tensor and modified constitutive tensor that incorporate the aging viscoelastic properties of the concrete, respectively.

For the numerical implementation purposes, relationship (7) may conveniently be written in incremental form:

Δ𝝈 = 𝑫 ∶ Δ𝜺 − 𝑫 ∶ Δ𝜺𝑠ℎ − 𝑫∗ ∶ Δ𝜺𝑐𝑟	(8)

As mentioned above, isotropic formulation is considered for shrinkage, so that increment of shrinkage strain reads:

Δ𝜺𝑠ℎ = Δ𝜀𝑠ℎ(𝑡𝑠)𝟏	(9) where 𝑡𝑠 represents the concrete curing time, and Δ𝜀𝑠ℎ is the variation in magnitude of the concrete deformation associated with shrinkage (the dependency Δ𝜀𝑠ℎ of on current time is omitted). The latter expression is determined based on CEB-FIP MC90 standard specifications [8]. The increment of creep strain Δ𝜺𝑐𝑟 is computed making use of the incremental algorithm developed by Bažant and Prasannan [3; 4], together with a model calibration that incorporates CEB-FIP MC90 standard formulation [8]. The latter procedure for model parameters identification is achieved by comparing the creep functions provided in references [3; 4] and [8], leading to the following equivalence:

0	𝑐 0	0	𝑐	0	1	𝜙0 and	1	(10)

𝐸 = 𝐸 (𝑡 ),	𝛾 (𝑡 − 𝑡 ) = 𝛽 (𝑡 − 𝑡 ),	=	→ 0

𝑣(𝑡)	𝐸𝑐𝑖	𝜂(𝑡)

in which 𝑡 refers to the current time value and 𝑡0 to the concrete age at the instant of load application (time interval 𝑡 − 𝑡0 is generally referred to as loading time or loading age). In Bažant and Prasannan [3; 4] model, 𝐸0 is the instantaneous elasticity modulus of the concrete formed aggregates and cement paste particles, 𝛾 (𝑡 − 𝑡0) is the microviscoelastic deformation of the volume fraction 𝑣(𝑡) of solidified concrete and 𝜂(𝑡) is the apparent macroscopic viscosity. In the CEB-FIP MC90 formulation [8], 𝐸𝑐(𝑡0) stands for the tangent elastic modulus of concrete at the instant of the loading application 𝑡0, 𝛽𝑐(𝑡 − 𝑡0) is a coefficient that depends on the loading age 𝑡 − 𝑡0, 𝜙0(𝑡0) is a coefficient depending on the age 𝑡0 of the concrete and 𝐸𝑐𝑖 represents the tangent elasticity modulus of the concrete at the age of 28 day.

Foi ajustado, mas Samir modificará essa parte em verde acima.

## 5. Spatial and time discretization of the domain

The geometric layout of the modeled domain Ω consists of a twin deep tunnel with a transverse gallery, as shown

in Fig. 4.

Figure 4: Geometric layout of the domain

This domain is parameterized based on the radius of the longitudinal tunnel 𝑅𝑖. The geometric parameters and boundary conditions for the domain problem are shown in Fig. 5. We considered front, side, and bottom symmetry to reduce computational cost. In this domain, 𝑑1 is the distance between longitudinal tunnels axes, 𝐿2 total excavated length, 𝑑3 domain height, 𝐿1 length of the unexcavated region, 𝐿3 transversal length of the domain, 𝐿𝑝 step length of the excavation process, 𝑑2 position of the gallery along the longitudinal tunnel. Together with boundary pressure

𝜎𝑥, 𝜎𝑦 and 𝜎𝑧, we apply the initial stress condition 𝝈0 = −𝜎𝑥𝒆𝑥 ⊗ 𝒆𝑥 − 𝜎𝑦𝒆𝑦 ⊗ 𝒆𝑦 − 𝜎𝑧𝒆𝑧 ⊗ 𝒆𝑧 at all integration points to simulate the initial state of the rock mass. The spatial discretization in Fig. 5 corresponds to a mesh with trilinear hexahedral elements (SOLID 185, 8 nodes), except in the gallery region, which uses higher-order tetrahedral elements (SOLID186, 10 nodes). We divided the mesh into two regions: one near the tunnel (light gray), which we refined more,

Figure 5: Mesh, dimensions and boundary conditions of the 3D twin tunnel domain

and a region farther away (dark gray), which we increased the aspect ratio to minimize the number of elements in that region. Due to the low deformation gradient away from the tunnel wall, elements in this area can be considerably larger than in other regions. Fig. 6 presents the mesh at the cross-section of the longitudinal tunnel, with 𝑒 representing the thickness of the lining.

d

1

2

x

y

Figure 6: Detail 1 - Mesh in longitudinal tunnel cross-section with spacing 𝑑1 =4𝑅𝑖

One of the aspects investigated in this work is the influence of the spacing 𝑑1 in the convergence of the longitudinal twin tunnel. Fig. 7 and Fig. 8 illustrate the spatial discretization in the gallery region and its connection with the longitudinal tunnel considering spacings 𝑑1 = 16𝑅𝑖, 8𝑅𝑖 and 4𝑅𝑖, respectively. We adopt the radius of the gallery as 2∕3𝑅𝑖, and its lining has the same material and thickness as the longitudinal tunnel. The dimensions 𝑑5 and 𝑑1 define the size of the transition region comprising tetrahedral elements between the gallery and the rest of the domain. Fig. 9 shows half of this transition region inside the rock mass.

Figure 7: Detail 2 - Side view of the mesh in gallery region with 𝑑1 =16𝑅𝑖, 𝑑1 =8𝑅𝑖 and 𝑑1 =4𝑅𝑖

Figure 8: Detail 3 - Bottom view of the mesh in gallery region with 𝑑1 =16𝑅𝑖, 𝑑1 =8𝑅𝑖 and 𝑑1 =4𝑅𝑖

Figure 9: Detail 4 - Isometric view of the portion of the mesh in gallery transition region 𝑑1 =16𝑅𝑖, 𝑑1 =8𝑅𝑖 and 𝑑1 =4𝑅𝑖

Fig. 10 shows the mesh of the lining at the junction of the gallery and the longitudinal tunnel for 𝑑1 = 4𝑅𝑖, 8𝑅𝑖, and 16𝑅𝑖. One noteworthy characteristic of this mesh is that it confines the tetrahedral elements within the contour of every excavation step.

Figure 10: Isometric view of the lining at the intersection for 𝑑1 =16𝑅𝑖, 𝑑1 =8𝑅𝑖 and 𝑑1 =4𝑅𝑖 - expansion of symmetry in the 𝑥𝑧 plane

The construction process is simulated through the deactivating and activating method, i.e., in each step of excavation, reducing the stiffness of the excavated element (multiply by 1E-8) and active the lining elements at a distance 𝑑0 from the excavation face (unlined length). With each excavation step, we execute the solution, and time advances based on the expression 𝑡𝑝 = 𝐿𝑝∕𝑉𝑝, where 𝐿𝑝 represents the length of the excavation step, and 𝑉𝑝 is the speed of the excavation face. Fig. 11 illustrates a schematic of the excavation process where 𝑛𝑝 is the number of excavation steps. In this Figure, 𝑛𝑝𝑖𝑔 represents the number of steps excavated in the longitudinal tunnel that starts gallery excavation. Once reaching this step, we pause the excavation of the longitudinal tunnel, and the gallery excavation begins. In the gallery, 𝐿𝑝1 is the step length of the gallery excavation, 𝑉𝑝1 is the speed of the gallery excavation, and 𝑑01 is the unlined length of the gallery. After completing the gallery excavation, the longitudinal tunnel excavation resumes. These parameters related to the geometry domain, excavation and installation of the lining are shown in Table 1.

Figure 11: Schematic of the excavation process

Table 1

Parameters related to the geometry of the domain, excavation and installation of the lining

| PARAMETERS SYMBOL |  |  | UNIT |  | VALUES |  |
| --- | --- | --- | --- | --- | --- | --- |
| Longitudinal tunnels |  |  |  |  |  |  |
| Radius of the longitudinal tunnel | 𝑅𝑖 |  | m |  | 𝑅𝑖 |  |
| Thickness of the lining | 𝑒 |  | m |  | 0.1𝑅𝑖 |  |
| Step length of the excavation process | 𝐿𝑝 |  | m |  | 1∕3𝑅𝑖 |  |
| Unlined length | 𝑑0 |  | m |  | 2𝐿𝑝 |  |
| Speed of the excavation face | 𝑉𝑝 |  | m/day |  | 12.5 |  |
| Excavation step time | 𝑡𝑝 |  | day |  | 𝐿𝑝∕𝑉𝑝 |  |
|  | Gallery |  |  |  |  |  |
| Radius of the gallery | 𝑅𝑖1 |  | m |  | 2∕3𝑅𝑖 |  |
| Thickness of the concrete lining | 𝑒1 |  | m |  | 0.1𝑅𝑖 |  |
| Step length of the excavation process 1 | 𝐿𝑝1 |  | m | 0.3𝑅𝑖1 | 0.3214𝑅𝑖1 0.3387𝑅𝑖1 |  |
| Unlined length | 𝑑01 |  | m |  |  | 2𝐿𝑝1 |
| Speed of the excavation face | 𝑉𝑝1 |  | m/day |  |  | 12.5 |
| Number of steps that starts gallery excavation |  | 𝑛𝑝𝑖𝑔 | un |  |  | 15 |
| Rest of domain |  |  |  |  |  |  |
| Distance between longitudinal tunnel axes |  | 𝑑1 | m | 4𝑅𝑖 |  | 8𝑅𝑖 16𝑅𝑖 |
| Length of the unexcavated region |  | 𝐿1 | m |  |  | 10𝑅𝑖 |
| Total excavated length |  | 𝐿2 | m | 100𝐿𝑝 |  |  |
| Domain height |  | 𝐿3 | m | 20𝑅𝑖 |  |  |

1

𝐿𝑝1 ≈1∕3𝑅𝑖1 in such a way that there are 𝑛 integer excavation steps in 𝑑1 −2𝑅𝑖

tunnel) and

During the tunnel construction phase, we adopted the initial time increment for the solution steps as $0.5t_p$ (during the excavation of the longitudinal tunnels) and $0.5t_{p_1}$ (during the excavation of the transverse gallery). ANSYS manages the time increment using the bisection method, halving the time step if there is no equilibrium convergence. After tunnel excavation, in time-dependent analyses, time continues to evolve in order to capture long-term viscous effects. At this stage, each time step lasts 100 days, with an initial increment of 50 days. This increase, compared to the time increments during excavation, is facilitated by the semi-implicit scheme in the viscoplasticity solution. The explicit scheme, as indicated in [\citenum{zienkiewicz1974visco}], requires a smaller time increment for the accuracy of the solution. The long term is determined when the increment of displacements between successive time intervals is less than 1E-8. This long-term time depends on the parameters of the rock mass and the lining.